# Camera Operator

### Video Production Team

### Position Handbook

## Terminology

Pan 	Horizontal movement of the camera left or right

Tilt	Vertical movement of the camera up and down

Zoom	Using thumb controls or by rotating the zoom control on the rounded part of the camera to enlarge (zoom in) or shrink (zoom out) the subject.

Framing	Adjusting the subject to a certain place in the camera viewing area

Lead Room	When viewing a subject from the side, the space in front of the subjects face

Head Room	Space above the subject’s head – less for close shots, more for wider shots.

Rule of Thirds

Imagine that there is a 3x3 box on the camera viewing screen to “frame” the subject so that they are ideally placed within the viewing area (not just centered).

On closer shots, the subject’s ears should be found on either the left or right-side vertical lines, depending on which way they are facing.

Example: For a tighter shot, if the subject is facing left to right in the shot, they should be framed so their ears are on or near the upper left intersection of the lines to provide them with more “lead” room (more room to their front than behind them). Center (“head on”) shots can center the subject.

Examples below

From right

From Left
 

From Center

## Camera Shot Names

Cover Shot – zoom out as far as you can covering a lot of the room.

Full Body – From head to toe of the person

Knee Up – From the subjects knees up

Waist up – From the subjects waist up

Chase – Keep up with the subject

Live – This means your camera is on air.

Preview – This means the video Director has your camera to go live next

Full Body Shot

Knee Up Shot

Waist Up Shot